Computer Networks Notes

UNIT 1 — INTRODUCTION TO COMPUTER NETWORKS

1. Computer Networks

Definition

A computer network is a collection of devices interconnected through communication links to share resources and exchange information.

Devices connected to the network are called nodes.

Examples of nodes:

- Computers

- Servers

- Printers

- Routers

- Smartphones

Network Components (Network Devices)

Network components are hardware devices used to connect computers and enable communication within a network.

They help in:

- data transmission

- signal amplification

- routing data

- connecting networks

Common network devices include:

- Hub

- Switch

- Router

- Bridge

- Repeater

- Gateway

- Modem

- Network Interface Card (NIC)

1. Hub

A hub is a networking device that connects multiple computers in a LAN and broadcasts data to all connected devices.

It works at the Physical Layer (Layer 1) of the OSI model.

How it works

When a device sends data to the hub, the hub sends that data to every device connected to it.

So all devices receive the data even if it is not meant for them.

Types of Hubs

- Active Hub

- Passive Hub

Advantages

- Simple device

- Cheap

- Easy to install

- Used in small networks

Disadvantages

- High data collision

- Low efficiency

- No intelligent data forwarding

- Security issues

2. Switch

A switch is a network device that connects multiple devices in a LAN and forwards data only to the intended device.

It operates at the Data Link Layer (Layer 2).

How it works

A switch uses MAC addresses to send data to the correct device instead of broadcasting it to everyone.

Advantages

- Reduces data collision

- Faster than hubs

- Better network efficiency

- Improved security

Disadvantages

- More expensive than hubs

- Complex configuration

- Requires management in large networks

Difference Between Hub and Switch

| Feature | Hub | Switch |
| --- | --- | --- |
| Layer | Physical Layer | Data Link Layer |
| Data Transmission | Broadcast to all devices | Sends to specific device |
| Speed | Slower | Faster |
| Collision | High collision | Very low collision |
| Intelligence | Dumb device | Intelligent device |

Key exam line:
A hub broadcasts data to all devices, whereas a switch forwards data only to the intended device using MAC addresses.

3. Router

A router is a network device used to connect multiple networks and route data packets between them.

It works at the Network Layer (Layer 3).

Routers use IP addresses to determine the best path for data transmission.

Example:
Your home WiFi router connects your LAN to the Internet.

Advantages

- Connects multiple networks

- Intelligent routing

- Improves security

- Reduces network traffic

Disadvantages

- Expensive

- Slower than switches in LAN communication

- Requires configuration

4. Bridge

A bridge connects two LAN segments and filters traffic between them.

It works at the Data Link Layer.

Functions

- Reduces network congestion

- Filters unnecessary traffic

Advantages

- Improves network performance

- Simple device

- Reduces traffic

Disadvantages

- Slower than switches

- Cannot connect different network types

5. Repeater

A repeater regenerates weak signals so they can travel longer distances.

Works at the Physical Layer.

Example:
Used in long-distance networks.

Advantages

- Extends network range

- Simple device

- Low cost

Disadvantages

- Cannot filter traffic

- Only amplifies signals

6. Gateway

A gateway connects two networks that use different protocols.

It acts as a protocol converter.

Example:
Connecting a TCP/IP network to another protocol network.

Advantages

- Connects different networks

- Protocol translation

- Flexible communication

Disadvantages

- Expensive

- Complex device

7. Modem

Full form: Modulator-Demodulator

A modem converts digital signals to analog signals and vice versa.

Used for internet communication over telephone lines.

Function

Digital → Analog (modulation)
Analog → Digital (demodulation)

8. NIC (Network Interface Card)

A NIC is a hardware component that allows a computer to connect to a network.

Each NIC has a unique MAC address.

Types

- Wired NIC

- Wireless NIC

Goals of Computer Networks

1. Resource Sharing

Networks allow multiple users to share:

- Hardware (printers, scanners)

- Software

- Files and databases

Example: A single printer used by many computers.

2. High Reliability

If one system fails, another system can take over.

Example: Backup servers.

3. Cost Reduction

Instead of buying separate devices for each computer, resources can be shared.

Example: One printer for an entire office.

4. Communication

Networks allow users to communicate through:

- Email

- Video conferencing

- Messaging systems

5. Scalability

Networks can easily expand by adding new devices.

Applications of Computer Networks

Business

- Online transactions

- Database access

- Cloud computing

Home Use

- Internet browsing

- Streaming

- Gaming

Mobile Users

- Smartphones

- Wireless internet

Government and Banking

- Online banking

- Digital records

- E-governance

Data Communication

Definition

Data communication is the exchange of data between two or more devices through a transmission medium.

Components of Data Communication

- Sender
Device that sends the data.

- Receiver
Device that receives the data.

- Message
Information being transmitted.

- Transmission Medium
Physical path used for communication.

Examples:

- Copper cables

- Fiber optics

- Wireless signals

- Protocol
Rules that control communication.

Data Flow Types

Simplex

Data flows in one direction only.

Example:
Keyboard → Computer

Half Duplex

Data flows in both directions but not at the same time.

Example:
Walkie-talkie

Full Duplex

Data flows in both directions simultaneously.

Example:
Telephone conversation

Network Connections

Point-to-Point Connection

A direct communication link between two devices.

Characteristics:

- Dedicated connection

- High speed

- More secure

Example:
Connection between two computers using a cable.

Multipoint Connection

A single communication link shared by multiple devices.

Characteristics:

- Cost-effective

- Shared bandwidth

Example:
Several computers connected to one network cable.

Network Topologies

Topology refers to the physical or logical arrangement of devices in a network.

Bus Topology

All devices share a single backbone cable.

Advantages

- Low cost

- Easy installation

- Less cable required

- Suitable for small networks

Disadvantages

- High chance of data collision

- Backbone failure stops entire network

- Performance decreases with many devices

- Difficult troubleshooting

Star Topology

All devices connect to a central hub or switch.

Advantages

- Easy to manage

- Easy to add new devices

- Failure of one device does not affect others

- Easy fault detection

Disadvantages

- Hub failure stops network

- More cable required

- Higher cost

- Performance depends on central device

Ring Topology

Devices connected in circular form.

Advantages

- No collision due to token passing

- Equal access for all nodes

- Good performance under heavy load

- Predictable transmission

Disadvantages

- Failure of one device affects network

- Difficult to add/remove devices

- Troubleshooting is difficult

- More expensive than bus

Mesh Topology

Each device connects to every other device.

Advantages

- Highly reliable

- No data collision

- High security

- Fault tolerant

Disadvantages

- Expensive

- Complex installation

- Requires large cabling

- Difficult maintenance

Categories of Networks

Based on geographic coverage.

1. LAN — Local Area Network

Local Area Network

A LAN connects devices within a small area like:

- Home

- Office

- School

- College lab

Example

Wi-Fi in your house or a college computer lab.

Advantages

- Very fast data transfer

- Cheap to set up

- Easy file and printer sharing

- Better security because it’s a private network

Disadvantages

- Limited range

- If main device/server fails, network may stop

- Requires maintenance

2. PAN — Personal Area Network

Personal Area Network

A PAN connects devices around one person within a very short range.

Example

- Bluetooth between phone and earbuds

- Smartwatch connected to phone

Advantages

- Simple setup

- Wireless and convenient

- Low power consumption

Disadvantages

- Very short range

- Slower speed compared to LAN

- Limited number of devices

3. MAN — Metropolitan Area Network

Metropolitan Area Network

A MAN covers a city or large campus.

Example

A network connecting multiple bank branches across Pune.

Advantages

- Covers larger area than LAN

- High-speed communication between locations

- Useful for large organizations

Disadvantages

- Expensive infrastructure

- More difficult to manage

- Security becomes harder

4. WAN — Wide Area Network

Wide Area Network

A WAN connects devices across countries or the world.

Example

The Internet.

Advantages

- Global connectivity

- Remote access from anywhere

- Supports massive communication

Disadvantages

- Expensive setup and maintenance

- Lower speed compared to LAN

- Higher security risks

- More latency

Protocols

Definition

A protocol is a set of rules that governs how data is transmitted between devices in a network.

Protocols define:

- Data format

- Transmission method

- Error handling

- Synchronization

Examples:

- HTTP - HyperText Transfer Protocol is used to transfer web pages from a web server to a web browser and HTTPS is a secure version which encrypts data using SSL/TLS

- FTP – File Transfer Protocol is used when sending files between a client and a server over a network.

- TCP – Transmission Control Protocol used in connection-oriented networks and ensures reliable and ordered delivery of data between devices.

- IMAP – Internet Message Access Protocol allows users to access emails stored on the mail server without downloading them permanently.

- POP3 – Post Office Protocol is used to receive emails from a mail server.

- SMTP – Simple Mail Transfer Protocol is used to send emails between mail servers.

- IP – Internet Protocol provides logical addressing and routing for data packets across networks.

- UDP – User Datagram Protocol is used in connectionless networks and sends data without establishing a connection

- ARP – Address Resolution Protocol converts IP address into a MAC address within a local network

- DNS – Domain Name System converts domain names into IP.

TCP vs UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Reliability | Reliable | Unreliable |
| Speed | Slower | Faster |
| Example Use | Web browsing | Gaming |

UNIT 2 – NETWORK MODELS

Network models are conceptual frameworks used to understand how data travels through a network.
They divide the communication process into layers, where each layer performs a specific function.

Two main models:

- OSI Reference Model

- TCP/IP Reference Model

1. OSI REFERENCE MODEL

OSI = Open Systems Interconnection Model

Developed By

ISO (International Organization for Standardization)

Purpose

The OSI model was created to standardize communication between different computer systems and networking devices.

Before this model, different companies used their own communication systems, which made interconnection difficult.

The OSI model solves this by dividing networking into 7 layers, each responsible for a specific task.

The 7 Layers of OSI Model

The layers are arranged from bottom to top.

- Physical Layer

- Data Link Layer

- Network Layer

- Transport Layer

- Session Layer

- Presentation Layer

- Application Layer

Mnemonic to remember them:

All People Seem To Need Data Processing

1. Physical Layer

This is the lowest layer of the OSI model.

Main Function

Transmission of raw bits (0s and 1s) over the physical medium.

Responsibilities

- Defines cables and connectors

- Electrical signal transmission

- Data transmission rate

- Physical network topology

Devices

- Hub

- Repeater

- Network cables

Example: Ethernet cables sending signals.

2. Data Link Layer

This layer ensures error-free data transfer between two directly connected nodes.

Main Functions

Framing
Divides data into frames.

Error Detection
Detects errors using checksums.

Physical Addressing
Uses MAC addresses to identify devices.

Flow Control
Controls data transmission speed.

Devices

- Switch

- Bridge

Example: Ethernet communication.

3. Network Layer

Responsible for routing data packets between networks.

Main Functions

Logical Addressing
Assigns IP addresses.

Routing
Determines the best path for data.

Packet forwarding

Device

Router

Protocol

IP (Internet Protocol)

Example: Data traveling across the internet.

4. Transport Layer

Responsible for end-to-end communication between devices.

Main Functions

Segmentation
Breaks large messages into smaller segments.

Error Control

Flow Control

Reliable Delivery

Protocols

TCP
UDP

Example: Reliable transfer of files.

5. Session Layer

Responsible for establishing, maintaining, and terminating communication sessions between devices.

Functions

- Session management

- Synchronization

- Dialogue control

Example: Login sessions in applications.

6. Presentation Layer

Responsible for data translation and formatting.

Functions

Encryption
Protects data security.

Compression
Reduces data size.

Data format conversion

Example: Converting ASCII to binary.

7. Application Layer

This is the top layer and closest to the user.

It provides network services directly to applications.

Examples of protocols

HTTP
FTP
SMTP
DNS

Example: Web browsing.

Why the OSI Model Is Important

- Standardizes networking processes

- Makes network troubleshooting easier

- Allows interoperability between systems

- Helps understand network architecture

Data Encapsulation in OSI

As data moves down layers:

Data → Segment → Packet → Frame → Bits

Each layer adds a header with control information.

TCP/IP MODEL

Transmission Control Protocol / Internet Protocol

Developed By

DARPA (U.S. Department of Defense)

Purpose

Designed specifically for internet communication.

Unlike OSI, TCP/IP is a practical model actually used in real networks.

TCP/IP Layers

TCP/IP has 4 layers.

- Application Layer

- Transport Layer

- Internet Layer

- Network Access Layer

1. Application Layer

Combines the functions of OSI application, presentation, and session layers.

Protocols

HTTP
FTP
SMTP
DNS

2. Transport Layer

Responsible for data transmission between devices.

Protocols

TCP
UDP

Functions:

- segmentation

- reliability

- flow control

3. Internet Layer

Responsible for logical addressing and routing.

Protocol used:
IP

Devices:
Routers

4. Network Access Layer

Handles communication with the physical network hardware.

Includes functions of:

- Data Link layer

- Physical layer

OSI vs TCP/IP Comparison

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Layers | 7 layers | 4 layers |
| Developer | ISO | DARPA |
| Type | Conceptual model | Practical model |
| Usage | Learning and design | Internet communication |
| Structure | Strict layering | Flexible layering |

Mapping OSI to TCP/IP

| OSI Layer | TCP/IP Layer |
| --- | --- |
| Application | Application |
| Presentation | Application |
| Session | Application |
| Transport | Transport |
| Network | Internet |
| Data Link | Network Access |
| Physical | Network Access |

Addressing in Computer Networks

Addressing is used to identify devices and applications in a network.

Three main types:

- Physical Address

- Logical Address

- Port Address

1. Physical Address

Also called MAC Address.

Characteristics

- Unique identifier for each network device

- 48-bit address

- Assigned to Network Interface Card

Example:

00:1A:2B:3C:4D:5E

Used in Data Link Layer.

2. Logical Address

Also called IP Address.

Used to identify devices across networks.

Example

192.168.1.1

Two versions:

IPv4 – 32 bit
IPv6 – 128 bit

Used in Network Layer.

3. Port Address

Port addresses identify specific applications or services running on a device.

Examples:

HTTP → Port 80
HTTPS → Port 443
FTP → Port 21
SMTP → Port 25

Used in Transport Layer.

Key Concept

Think of networking like sending a letter.

MAC Address → House address
IP Address → City address
Port Address → Specific person in the house

UNIT 3 — PHYSICAL LAYER

What is the Physical Layer?

The Physical Layer is the lowest layer of the OSI model.

Main Role

It is responsible for transmitting raw bits (0s and 1s) through a physical medium such as cables, fiber optics, or wireless signals.

In simple terms:
This layer converts digital data into electrical, optical, or radio signals and sends them through the communication channel.

Responsibilities of the Physical Layer

The physical layer defines the physical characteristics of communication.

Major responsibilities include:

1. Bit Transmission

It transmits binary data (0 and 1) over the communication channel.

2. Physical Topology

Defines how devices are physically connected in the network.

Examples:

- Bus

- Star

- Ring

3. Data Rate

Determines how many bits can be transmitted per second.

Example:
100 Mbps Ethernet.

4. Synchronization

Ensures sender and receiver are synchronized when transmitting bits.

5. Transmission Mode

Defines the direction of data flow between devices.

Signals

Data travels through networks in the form of signals.

A signal is an electromagnetic or electrical representation of data used for communication.

There are two main types of signals.

Analog Signals

Analog signals are continuous signals that vary smoothly over time.

Characteristics:

- Infinite number of values

- Continuous waveform

Examples:

- Radio signals

- Voice transmission

- Television signals

Example waveform: sine wave.

Digital Signals

Digital signals represent data using discrete values, usually 0 and 1.

Characteristics:

- Finite number of values

- Square-shaped waveform

Digital signals are used in:

- Computers

- Digital communication systems

- Ethernet networks

Transmission Modes

Transmission mode defines how data flows between devices.

Simplex Transmission

Data flows in only one direction.

Example:
Keyboard → Computer.

Half Duplex Transmission

Data flows in both directions but not simultaneously.

Example:
Walkie-talkies.

Full Duplex Transmission

Data flows in both directions at the same time.

Example:
Telephone conversation.

Parallel Transmission

Parallel transmission sends multiple bits simultaneously using multiple channels or wires.

Example:
Older printer cables.

Advantages:

- Faster for short distances.

Disadvantages:

- Expensive

- Signal interference

- Not suitable for long distances

Serial Transmission

Serial transmission sends bits one after another through a single communication channel.

Example:
USB communication
Ethernet networks

Advantages:

- Reliable

- Suitable for long-distance transmission

- Less interference

Line Encoding

What is Line Encoding?

Line encoding is the process of converting digital data into digital signals for transmission over a communication/computer medium.

The computer generates binary data, but it must be converted into voltage signals before transmission.

Purpose:

- Maintain synchronization

- Reduce errors

- Improve signal efficiency

Types of Line Encoding

Three main categories.

- Unipolar Encoding

- Polar Encoding

- Bipolar Encoding

Unipolar Encoding

Uses only one voltage level.

Representation:

- 1 → positive voltage

- 0 → zero voltage

Example:
Unipolar NRZ.

Disadvantages:

- Synchronization problems

- DC component present

- Poor efficiency

Because of these issues, unipolar encoding is rarely used.

Polar Encoding

Polar encoding uses two voltage levels.

Positive voltage and negative voltage represent binary data.

Common polar encoding schemes include:

- NRZ-L

- NRZ-I

- Manchester

- Differential Manchester

NRZ-L (Non Return to Zero-Level)

In NRZ-L:

- One voltage level represents 1

- Another voltage level represents 0

Problem:
Long sequences of identical bits create synchronization issues.

NRZ-I (Non Return to Zero-Invert)

In NRZ-I:

- Signal transition represents binary 1

- No transition represents binary 0

This method improves synchronization.

Manchester Encoding

Very important encoding technique.

Each bit contains a transition in the middle.

Representation:

- 0 → high to low transition

- 1 → low to high transition

Advantages:

- Self-synchronizing

- No DC component

- Reliable timing recovery

Disadvantage:
Requires double bandwidth.

Used in Ethernet networks.

Differential Manchester Encoding

Similar to Manchester encoding.

Rules:

- Always a transition in the middle

- Beginning transition determines bit value

Advantages:

- Good synchronization

- Less sensitive to polarity changes

Bipolar Encoding

Bipolar encoding uses three voltage levels.

- Positive voltage

- Negative voltage

- Zero voltage

Example:
AMI (Alternate Mark Inversion).

Rules:

- 0 → zero voltage

- 1 → alternating positive and negative voltage

Advantages:

- No DC component

- Efficient signal transmission

Disadvantage:
More complex implementation.

Digital-to-Analog Conversion

Sometimes digital data must be transmitted using analog signals.

This process is called modulation.

Three common techniques are used.

ASK (Amplitude Shift Keying)

In ASK, the amplitude of the signal changes to represent binary data.

Example:
High amplitude → 1
Low amplitude → 0

Used in simple communication systems.

FSK (Frequency Shift Keying)

In FSK, the frequency of the signal changes to represent data.

Example:
High frequency → 1
Low frequency → 0

Used in modems and radio communication.

PSK (Phase Shift Keying)

In PSK, the phase of the signal changes to represent data.

Example:
Phase shift represents binary values.

PSK is more efficient and widely used in modern communication systems.

Analog-to-Analog Modulation

Used when analog signals are transmitted over analog channels.

Three techniques.

AM (Amplitude Modulation)

Amplitude of the carrier signal changes according to the message signal.

Used in:
AM radio broadcasting.

FM (Frequency Modulation)

Frequency of the carrier signal changes.

Used in:
FM radio transmission.

Advantages:
Better noise resistance than AM.

PM (Phase Modulation)

Phase of the carrier signal changes according to the message signal.

Used in advanced communication systems.

Physical Layer Devices

Several networking devices operate at the physical layer.

Repeater

A repeater regenerates and amplifies weak signals.

Purpose:
Extend network transmission distance.

Example:
Long-distance Ethernet networks.

Hub

A hub connects multiple devices in a network and broadcasts data to all connected devices.

Characteristics:

- Works at Physical Layer

- Causes collisions

- Used in star topology

Active Hub

An active hub amplifies and regenerates signals.

Requires power supply.

Passive Hub

A passive hub simply connects devices without amplifying signals.

Does not require power.

Why Physical Layer Matters

Without the physical layer:

- Data cannot travel through cables

- Devices cannot communicate

- Networks cannot function

It forms the foundation of all network communication.

Quick High-Value Points for Exam

Remember these:

Physical layer → transmits bits
Analog signal → continuous
Digital signal → discrete
Serial transmission → one bit at a time
Parallel transmission → multiple bits simultaneously
Manchester encoding → transition in middle of bit
ASK/FSK/PSK → digital-to-analog modulation

COMPUTER NETWORKS NOTES

UNIT 4 — THE DATA LINK LAYER

What is the Data Link Layer?

The Data Link Layer is the second layer of the OSI model.

Main Role

It is responsible for reliable node-to-node delivery of data between directly connected devices.

The physical layer only sends raw bits.
The Data Link Layer ensures:

- proper framing

- error detection

- flow control

- reliable communication

It converts raw bits into organized units called frames.

Responsibilities of the Data Link Layer

1. Framing

The data link layer divides data received from the network layer into smaller units called frames.

A frame contains:

- Header

- Data

- Trailer

Purpose:

- organize transmission

- detect errors

- identify sender and receiver

2. Physical Addressing

Uses MAC addresses for communication between devices in the same network.

Example:
Every NIC card has a unique MAC address.

3. Error Control

Detects and sometimes corrects errors during transmission.

Techniques:

- CRC

- checksum

- parity checking

4. Flow Control

Controls the speed of data transfer between sender and receiver.

Purpose:
Prevent fast sender from overwhelming slow receiver.

5. Access Control

Determines which device can use the communication channel at a given time.

Important in shared networks.

Services Provided to the Network Layer

The Data Link Layer provides services to the Network Layer.

Three major services:

1. Unacknowledged Connectionless Service

- No acknowledgement is sent.

- No error recovery.

- Fast communication.

Used where reliability is not critical.

Example:
Real-time voice transmission.

Advantages:

- Fast

- Simple

- Low overhead

Disadvantages:

- Unreliable

- Data loss possible

2. Acknowledged Connectionless Service

Each frame is acknowledged individually.

If acknowledgement is not received, frame is retransmitted.

Advantages:

- Reliable

- Error recovery possible

Disadvantages:

- More overhead

- Slower than unacknowledged service

3. Acknowledged Connection-Oriented Service

A connection is established before transmission.

Characteristics:

- frames delivered in order

- reliable communication

- acknowledgements used

Advantages:

- Highly reliable

- Ordered delivery

Disadvantages:

- Complex

- More delay

Framing

What is Framing?

Framing is the process of dividing data into smaller units called frames.

Frames help:

- synchronization

- error detection

- proper delivery

Framing Methods

1. Character Count Method

The header contains the number of characters present in the frame.

Example:
If frame contains 20 bytes, header stores value 20.

Problem:
If count value becomes corrupted, receiver loses synchronization.

Advantages:

- Simple implementation

Disadvantages:

- One error can destroy entire framing structure

2. Flag Bytes with Byte Stuffing

Special bytes called flag bytes mark the beginning and end of a frame.

Common flag:
01111110

Problem

What if same flag appears inside data?

Solution: Byte Stuffing

An extra escape character is inserted before the flag appearing inside data.

Receiver removes the extra byte during decoding.

Advantages:

- Reliable framing

- Prevents confusion with flag bytes

Disadvantages:

- Extra overhead

3. Bit Stuffing

Used in bit-oriented protocols.

Frames begin and end with a special bit pattern.

Example flag:
01111110

Rule

Whenever five consecutive 1s appear in data, sender inserts an extra 0.

Receiver removes the stuffed 0.

Advantages:

- Efficient for bit-oriented transmission

- Reliable synchronization

Disadvantages:

- Slight increase in frame size

4. Physical Layer Coding Violations

Certain unused signal patterns are used to indicate frame boundaries.

Advantages:

- No additional stuffing required

- Efficient framing

Disadvantages:

- Depends on physical encoding method

Flow Control

What is Flow Control?

Flow control regulates the rate of data transmission.

Purpose:
Prevent receiver overflow.

Types of Flow Control

1. Stop-and-Wait Flow Control

Sender sends one frame and waits for acknowledgement before sending next frame.

Advantages:

- Simple

- Reliable

Disadvantages:

- Slow performance

- Wastes bandwidth

2. Sliding Window Flow Control

Multiple frames can be sent before receiving acknowledgement.

A window determines how many frames can be transmitted.

Advantages:

- Better efficiency

- Higher throughput

Disadvantages:

- More complex

Error Control

What is Error Control?

Error control ensures reliable data transmission by detecting and correcting errors.

Causes of errors:

- noise

- signal distortion

- interference

Error Detection Techniques

1. Parity Check

An extra parity bit is added.

Even Parity

Total number of 1s becomes even.

Odd Parity

Total number of 1s becomes odd.

Advantages:

- Very simple

Disadvantages:

- Cannot detect multiple errors effectively

2. Checksum

Data is divided into segments.
A sum is calculated and transmitted.
Receiver recalculates checksum.

If values differ, error is detected.

Used in:

- IP

- TCP

- UDP

3. CRC (Cyclic Redundancy Check)

Most important and widely used error detection method.

Working

- Data is treated as binary polynomial.

- Sender divides data by a generator polynomial.

- Remainder is attached to frame.

- Receiver performs same division.

If remainder becomes zero:
No error.

If remainder is non-zero:
Error detected.

Advantages:

- Very reliable

- Detects burst errors

- Widely used in networks

Disadvantages:

- More complex than parity check

Used in:

- Ethernet

- Wi-Fi

- Data communication systems

Important High-Value Exam Points

Remember these:

- Data Link Layer converts bits into frames

- Framing organizes data transmission

- Byte stuffing inserts escape characters

- Bit stuffing inserts 0 after five consecutive 1s

- Stop-and-Wait is simple but slow

- Sliding Window improves efficiency

- CRC is the most reliable error detection technique

UNIT 5 — THE MEDIUM ACCESS SUBLAYER

What is the Medium Access Sublayer?

The Medium Access Control (MAC) Sublayer is a part of the Data Link Layer.

Main Role

It controls how multiple devices share and access a common communication channel.

Problem:
When many devices try to send data at the same time, collisions may occur.

The MAC sublayer provides rules to:

- avoid collisions

- reduce congestion

- improve efficiency

Categories of MAC Protocols

Three major categories:

- Random Access Protocols

- Controlled Access Protocols

- Channelization Protocols

1. Random Access Protocols

In random access methods:

- no fixed control exists

- any device can transmit anytime

- collisions are possible

ALOHA

ALOHA is one of the earliest random access protocols.

Two types:

- Pure ALOHA

- Slotted ALOHA

Pure ALOHA

Devices transmit whenever data is available.

If collision occurs:

- wait random time

- retransmit

Advantages:

- Very simple

- Easy implementation

Disadvantages:

- High collision rate

- Poor efficiency

Maximum efficiency:
18.4%

Slotted ALOHA

Time is divided into slots.
Transmission allowed only at slot boundaries.

Advantages:

- Fewer collisions than Pure ALOHA

- Better efficiency

Disadvantages:

- Requires synchronization

Maximum efficiency:
36.8%

CSMA (Carrier Sense Multiple Access)

Before transmitting, a device checks whether channel is busy.

If channel is free:
Transmit.

If channel is busy:
Wait.

Advantages:

- Reduces collisions

- Better than ALOHA

Disadvantages:

- Collisions still possible because of propagation delay

Types of CSMA

1-Persistent CSMA

- Device continuously senses channel.

- Transmits immediately when channel becomes free.

Advantage:

- Fast transmission

Disadvantage:

- Higher collision probability

Non-Persistent CSMA

If channel is busy:

- device waits random time

- checks again later

Advantage:

- Fewer collisions

Disadvantage:

- More delay

P-Persistent CSMA

Used in slotted channels.

When channel becomes free:

- transmit with probability P

- wait with probability (1-P)

Advantage:

- Balanced performance

CSMA/CD (Carrier Sense Multiple Access with Collision Detection)

Used mainly in Ethernet networks.

Working

- Sense channel

- Transmit if free

- Detect collision during transmission

- Stop transmission if collision occurs

- Retransmit after random delay

Advantages:

- Efficient for wired LANs

- Reduces unnecessary transmission

Disadvantages:

- Not suitable for wireless networks

CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

Used in wireless networks like Wi-Fi.

Wireless devices cannot easily detect collisions.
Therefore they try to avoid collisions before transmission.

Techniques Used

- acknowledgements

- random backoff timers

- RTS/CTS mechanism

Advantages:

- Suitable for wireless communication

- Reduces collisions

Disadvantages:

- More overhead

- Slightly slower

2. Controlled Access Protocols

In controlled access:

- devices take turns transmitting

- collisions are avoided

Reservation

Devices reserve transmission slots before sending data.

Advantages:

- No collision

- Efficient under heavy traffic

Disadvantages:

- Reservation overhead

Polling

A central controller asks devices one by one whether they want to transmit.

Advantages:

- No collision

- Orderly communication

Disadvantages:

- Polling overhead

- Failure of controller affects entire system

Token Passing

A special frame called token circulates through network.
Only device holding token can transmit.

Advantages:

- Collision-free communication

- Predictable performance

Disadvantages:

- Token loss can stop communication

- Complex management

Used in:

- Token Ring networks

3. Channelization Protocols

Channelization divides bandwidth among users.

Three major methods:

- FDMA

- TDMA

- CDMA

FDMA (Frequency Division Multiple Access)

Available bandwidth divided into frequency bands.
Each user gets a separate frequency.

Advantages:

- Simultaneous communication possible

- Simple implementation

Disadvantages:

- Bandwidth wastage

- Guard bands required

Used in:

- Radio broadcasting

TDMA (Time Division Multiple Access)

Users share same frequency but different time slots.

Advantages:

- Better bandwidth utilization

- No frequency overlap

Disadvantages:

- Synchronization required

Used in:

- GSM networks

CDMA (Code Division Multiple Access)

All users share same frequency and time.
Each user gets unique code.

Advantages:

- High capacity

- Better security

- Resistant to interference

Disadvantages:

- Complex implementation

- Expensive

Used in:

- 3G mobile networks

Important High-Value Exam Points

Remember these:

- ALOHA allows random transmission

- CSMA senses channel before transmission

- CSMA/CD detects collisions

- CSMA/CA avoids collisions

- Token passing is collision free

- FDMA uses frequencies

- TDMA uses time slots

- CDMA uses unique codes

UNIT 6 — THE NETWORK LAYER

What is the Network Layer?

The Network Layer is the third layer of the OSI model.

Main Role

Responsible for:

- logical addressing

- routing

- packet forwarding

- inter-network communication

It determines the best path for data transmission.

Device used:
Router

Protocol used:
IP (Internet Protocol)

Functions of the Network Layer

1. Logical Addressing

Provides IP addresses to devices.

Example:
192.168.1.1

2. Routing

Determines best path for packets.

Routers use routing tables.

3. Packet Forwarding

Moves packets from source to destination.

4. Fragmentation

Large packets are divided into smaller packets.

Required because different networks support different maximum frame sizes.

Store-and-Forward Packet Switching

Routers store entire packet before forwarding.

Steps

- Receive packet

- Store packet temporarily

- Check errors

- Forward to next router

Advantages:

- Reliable

- Error checking possible

Disadvantages:

- Delay introduced

Services Provided to the Transport Layer

Two major services:

1. Connectionless Service

Packets travel independently.

Characteristics:

- no setup required

- packets may take different routes

- fast and simple

Example:
IP protocol

Advantages:

- Efficient

- Flexible

Disadvantages:

- No guarantee of delivery

- Packets may arrive out of order

2. Connection-Oriented Service

A path is established before transmission.

Characteristics:

- ordered delivery

- reliable communication

- connection setup required

Advantages:

- Reliable

- Predictable communication

Disadvantages:

- More overhead

- Slower setup

Virtual Circuit vs Datagram Network

| Feature | Virtual Circuit | Datagram |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Path | Fixed path | Different paths possible |
| Reliability | Higher | Lower |
| Packet Order | Maintained | May change |
| Setup Required | Yes | No |

Key exam line:
Virtual circuits use a fixed route, whereas datagram networks route each packet independently.

IP Addressing

IPv4 Address

IPv4 is a 32-bit logical address.

Example:
192.168.1.1

Structure:

- divided into 4 octets

- each octet = 8 bits

Total addresses:
2^32

IPv6 Address

IPv6 is a 128-bit address developed to solve IPv4 exhaustion.

Example:
2001:0db8:85a3:0000:0000:8a2e:0370:7334

Advantages:

- huge address space

- better security

- improved routing

Address Space

Address space means total number of unique addresses possible.

IPv4:
2^32 addresses

IPv6:
2^128 addresses

Address Notation

IPv4 uses dotted decimal notation.

Example:
192.168.10.5

IPv6 uses hexadecimal notation.

Example:
2001:db8::1

Classful Addressing

IPv4 originally used classes.

Class A

- First bit = 0

- Range: 1–126

- Large networks

Default subnet mask:
255.0.0.0

Class B

- First bits = 10

- Range: 128–191

- Medium networks

Subnet mask:
255.255.0.0

Class C

- First bits = 110

- Range: 192–223

- Small networks

Subnet mask:
255.255.255.0

Class D

Used for multicast.

Range:
224–239

Class E

Reserved for research.

Range:
240–255

Subnetting

Subnetting divides one network into smaller networks.

Purpose:

- better management

- reduced congestion

- efficient address usage

Advantages:

- Improved security

- Better performance

- Efficient IP allocation

Supernetting

Supernetting combines multiple smaller networks into one larger network.

Purpose:
Reduce routing table size.

Classless Addressing (CIDR)

CIDR = Classless Inter-Domain Routing

Uses variable-length subnet masks.

Example:
192.168.1.0/24

Advantages:

- efficient address allocation

- reduces wastage

IPv4 Datagram Format

An IPv4 datagram contains:

- Version

- Header Length

- Total Length

- Source Address

- Destination Address

- TTL

- Protocol

- Checksum

- Data

Fragmentation

Large packets may be fragmented into smaller packets.

Occurs when packet size exceeds MTU.

Important Fields

- Identification

- Fragment Offset

- More Fragment bit

Advantages:

- Allows communication across different networks

Disadvantages:

- Adds overhead

- Reassembly delay

Checksum

Checksum detects errors in IP header.

Receiver recalculates checksum.

If values differ:
Packet discarded.

Options Field

Provides optional control information.

Examples:

- security

- routing

- timestamps

Usually not heavily used because it increases overhead.

Important High-Value Exam Points

Remember these:

- Network layer handles routing and logical addressing

- Router works at Network Layer

- IPv4 = 32 bit

- IPv6 = 128 bit

- Subnetting divides networks

- CIDR enables classless addressing

- Datagram networks are connectionless

- Virtual circuits are connection-oriented

UNIT 7 — THE TRANSPORT LAYER

What is the Transport Layer?

The Transport Layer is the fourth layer of the OSI model.

Main Role

Provides end-to-end communication between applications running on different devices.

It ensures:

- reliable delivery

- segmentation

- flow control

- congestion control

Main protocols:

- TCP

- UDP

Functions of the Transport Layer

1. Segmentation

Large messages are divided into smaller segments.

Receiver reassembles them.

2. Service Point Addressing

Uses port numbers to identify applications.

Examples:

- HTTP → Port 80

- HTTPS → Port 443

- FTP → Port 21

3. Flow Control

Controls transmission speed between sender and receiver.

4. Error Control

Ensures reliable communication through acknowledgements and retransmission.

5. Congestion Control

Prevents excessive traffic in the network.

Transport Services

Two major services:

Connection-Oriented Service

Provided by TCP.

Characteristics:

- reliable

- ordered delivery

- acknowledgements used

Advantages:

- High reliability

- Error recovery possible

Disadvantages:

- Slower

- More overhead

Connectionless Service

Provided by UDP.

Characteristics:

- no connection setup

- no guarantee of delivery

- faster communication

Advantages:

- Fast

- Low overhead

Disadvantages:

- Unreliable

- No retransmission

Elements of Transport Protocols

Addressing

Transport layer uses port numbers.

Connection Establishment

Connection-oriented protocols establish connection before communication.

TCP uses three-way handshake.

Connection Release

Connection is terminated after communication.

Multiplexing

Multiple applications share same network connection.

Crash Recovery

Handles failures and retransmissions.

Congestion Control

What is Congestion?

Congestion occurs when network traffic exceeds capacity.

Effects:

- packet loss

- delay

- reduced performance

Causes of Congestion

- excessive traffic

- slow routers

- limited bandwidth

- retransmissions

Congestion Control Techniques

1. Traffic Shaping

Controls rate of data entering network.

Methods:

- Leaky Bucket

- Token Bucket

2. Admission Control

Network rejects new connections if congestion becomes severe.

3. Load Shedding

Some packets are discarded intentionally during congestion.

UDP (User Datagram Protocol)

UDP is a connectionless transport protocol.

Characteristics

- fast

- unreliable

- no acknowledgement

- low overhead

Advantages:

- Very fast

- Suitable for real-time applications

Disadvantages:

- No guaranteed delivery

- No flow control

Used in:

- online gaming

- video streaming

- DNS

- VoIP

TCP (Transmission Control Protocol)

TCP is a connection-oriented transport protocol.

Characteristics

- reliable

- ordered delivery

- acknowledgements used

- error recovery

Advantages:

- Reliable communication

- Error detection and retransmission

- Flow control supported

Disadvantages:

- Slower than UDP

- More overhead

Used in:

- web browsing

- email

- file transfer

TCP Three-Way Handshake

Used to establish connection.

Step 1 — SYN

Client sends SYN request.

Step 2 — SYN-ACK

Server replies with SYN-ACK.

Step 3 — ACK

Client sends ACK.

Connection established.

TCP vs UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Reliability | Reliable | Unreliable |
| Speed | Slower | Faster |
| Error Recovery | Yes | No |
| Acknowledgement | Used | Not used |
| Example Use | Web browsing | Gaming |

Performance Issues

Transport layer performance affected by:

- bandwidth

- delay

- congestion

- packet loss

- retransmissions

Important metrics:

- Throughput

- Latency

- Reliability

Important High-Value Exam Points

Remember these:

- Transport layer provides end-to-end communication

- TCP is reliable and connection-oriented

- UDP is fast and connectionless

- Congestion reduces network performance

- Three-way handshake establishes TCP connection

UNIT 8 — THE APPLICATION LAYER

What is the Application Layer?

The Application Layer is the topmost layer of both OSI and TCP/IP models.

Main Role

Provides network services directly to end users and applications.

Examples:

- web browsing

- email

- file transfer

- streaming

Functions of the Application Layer

- user interaction with network

- file transfer

- email communication

- remote login

- web services

- multimedia streaming

Domain Name System (DNS)

What is DNS?

DNS converts domain names into IP addresses.

Example:
www.google.com → IP address

Without DNS:
Users would need to remember numeric IP addresses.

Working of DNS

- User enters domain name.

- DNS request sent to DNS server.

- DNS server finds matching IP address.

- IP address returned to user.

- Browser connects to website.

Advantages of DNS

- Easy to remember names

- Faster access to websites

- Supports distributed databases

Electronic Mail (E-Mail)

Electronic mail allows users to send and receive digital messages.

Components of E-Mail System

1. User Agent

Software used to read and send emails.

Example:
Gmail
Outlook

2. Mail Server

Stores and forwards emails.

E-Mail Protocols

SMTP (Simple Mail Transfer Protocol)

Used for sending emails.

Default Port:
25

POP3 (Post Office Protocol Version 3)

Used for downloading emails from server.

Characteristics:

- emails downloaded locally

- server copy may be removed

IMAP (Internet Message Access Protocol)

Allows accessing emails directly from server.

Advantages:

- synchronization across devices

- emails remain on server

The World Wide Web (WWW)

The World Wide Web is a collection of interconnected web pages accessible through the Internet.

Main components:

- web browser

- web server

- HTTP protocol

HTTP (HyperText Transfer Protocol)

Used for communication between browser and web server.

Characteristics:

- request-response protocol

- stateless protocol

HTTPS

Secure version of HTTP.

Uses SSL/TLS encryption.

Advantages:

- secure communication

- protects passwords and financial data

Streaming Audio and Video

Streaming allows multimedia playback while data is still being received.

Examples:

- YouTube

- Netflix

- Spotify

Advantages:

- real-time playback

- no full download required

Disadvantages:

- requires stable internet

- buffering possible

Types of Streaming

Live Streaming

Real-time broadcasting.

Example:
Live sports.

On-Demand Streaming

Pre-recorded content available anytime.

Example:
Netflix movies.

Content Delivery Network (CDN)

A CDN is a distributed network of servers used to deliver content faster.

Copies of content are stored at multiple locations.

When user requests data:
Nearest server responds.

Advantages of CDN

- faster loading

- reduced server load

- lower latency

- better user experience

Disadvantages of CDN

- expensive infrastructure

- complex management

Common Application Layer Protocols

| Protocol | Full Form | Purpose |
| --- | --- | --- |
| HTTP | HyperText Transfer Protocol | Web browsing |
| HTTPS | HyperText Transfer Protocol Secure | Secure web communication |
| FTP | File Transfer Protocol | File transfer |
| SMTP | Simple Mail Transfer Protocol | Sending email |
| POP3 | Post Office Protocol | Receiving email |
| IMAP | Internet Message Access Protocol | Accessing emails |
| DNS | Domain Name System | Domain to IP conversion |

Important High-Value Exam Points

Remember these:

- DNS converts domain names to IP addresses

- SMTP sends emails

- POP3 downloads emails

- IMAP keeps emails on server

- HTTP is used for web communication

- HTTPS provides encryption

- CDN improves content delivery speed

- Streaming allows playback before full download